# Constraint-Based Production Optimizer

Randomized Scenario Benchmark — Allocation Under Material Scarcity

Summary. Across eight randomized, realistically-constrained plant scenarios, the optimizer raised sales-order fill by a median of +18 percentage points (range +9 to +43) and produced a median of 1.33× more feasible output (range 1.14–2.62×) from the same materials, versus conventional date-priority execution. All eight scenarios favored the optimizer; the advantage grows the more constrained the period.

# Method

A neutral synthetic plant (≈240 finished-good SKUs, multi-level BOM, shared raw materials, Pareto demand mix) was generated in JDE F-table format — sales orders, safety stock, forecast, BOMs, on-hand, and open POs.

Component supply coverage was drawn from a single neutral distribution (centered just above 1.0), so most materials are adequate and some fall short naturally — contention emerges from realistic BOM sharing, not from design.

Baseline (“no optimizer”): date-priority execution — earliest-due work orders consume the scarce material first, each made to its net requirement until the material runs out.

Optimizer: the engine at its default fill-rate setting, allocating the identical supply.

Eight scenarios were generated with different random seeds and evaluated on sales-order fill and feasible output.

# Results

| Scenario | SO fill — no optimizer | SO fill — optimizer | Δ (points) | Output × |
| --- | --- | --- | --- | --- |
| 1 | 66.6% | 75.5% | +8.9 | 1.14 |
| 2 | 61.8% | 71.4% | +9.5 | 1.15 |
| 3 | 57.7% | 74.8% | +17.1 | 1.32 |
| 4 | 58.8% | 71.1% | +12.3 | 1.22 |
| 5 | 28.3% | 71.2% | +42.9 | 2.62 |
| 6 | 46.7% | 77.3% | +30.6 | 1.65 |
| 7 | 54.8% | 73.8% | +18.9 | 1.33 |
| 8 | 40.8% | 67.3% | +26.5 | 1.65 |

Across 8 scenarios: sales-order-fill gain median +18.0 pts (range +8.9 to +42.9); feasible-output multiple median 1.33× (range 1.14–2.62×).

# Why it works

Finished goods vary in weight per case, so cases produced per pound of a scarce shared material differ across products. Date-priority execution fills whatever is due next and spends the scarce material on heavy cases; the optimizer steers it toward higher-yield products and matches production to demand priority (sales orders before safety stock before forecast before rounding up to plan). Total material is unchanged — only its allocation differs, which is where the additional orders and output come from.

# Scope & honest limits

These are synthetic benchmark results on representative constrained scenarios — not a measurement of any specific plant.

The baseline is pure date-priority allocation, a fair proxy for un-optimized automated execution; a skilled human planner would close part of the gap. The median (+18 pts) is more representative than the tightest-period outlier (+43).

Reported figures are the distribution across randomized draws, not a single tuned run.

Generated September 03, 2026 · JDE Production Optimizer